QUT Autopilot – Handover Documentation

Last Updated 6/09/2024.

Timeline:

24/06/2024: Received handover documents.

1/07/2024: Found issues with old PCB and made changes to the schematic on Altium.

10/07/2024: Began testing every major flight controller sensor through writing Arduino programs to view sensor data.

17/07/2024: Made more changes to PCB schematic as well as adding extra test points for key components.

29/07/2024: Ensured the PCB followed design rules for high frequency components and prepared Gerber files for new PCB order.

To Do:

Order new PCB from Altium schematic.

Buy remaining components missing.

Potentially make or buy breakout boards to easier test certain sensors within the circuit.

Solder up and test major sensors on new PCB.

Load Ardupilot firmware onto flight controller and begin customising.

Notes:

Sensor Testing

- STM32F - Microcontroller - Arduino 3.3V to Vin. Arduino GND to GND. Arduino Rx to Rx. Arduino Tx to Tx.

  - Input voltage 1.7 - 3.6V

- ICM-20948 - IMU - Arduino 3.3V to Vin. Arduino GND to GND. Arduino SCL to MPU9250 SCL. Arduino SDA to MPU9250 MOSI.

  - Input Voltage 1.7 - 3.6V, I/O pins requiring 1.71 - 1.95.

- ICM-20608-G - IMU - Arduino 3.3V to Vin. Arduino GND to GND. Arduino SCL to MPU9250 SCL. Arduino SDA to MPU9250 MOSI.

  - Both VDD/ VDDIO 1.71 - 3.45V

- MS56110 - Barometer - Arduino 5V to 5V in, Arduino 3.3v to 3.3V en, Arduino GND to GND. Arduino pin 13 to SCK, Arduino pin 12 to MISO, Arduino pin 11 to MOSI, Arduino pin 10 to CS.

  - VDD 1.8 - 3.6V

- LIS3MD - Magnetometer - Arduino 5V to 5V in, Arduino 3.3v to 3.3V en, Arduino GND to GND. Arduino pin 13 to SCK, Arduino pin 12 to MISO, Arduino pin 11 to MOSI, Arduino pin 10 to CS.

  - VDD 1.9 - 3.6v

- ESP8266 - Wi-Fi Module - Arduino pin 1 to ESP TX, Arduino pin 0 to ESP RX, Arduino 3.3v to ESP 8266_PD, Arduino 3.3v to ESP VCC, Arduino GND to ESP GND

Key Notes

- Arduino Uno powers the SPI bus with 5v while most of our IC’s SPI pins aren’t 5v tolerant causing issues that most likely cook the IC. Will likely need a bidirectional level shifter to step down voltages from each SPI pin or can use a voltage divider with resistors for testing purposes.

- SCLK voltage is usually low as Arduino pin 13 is broken, only outputs 0.2v. With a pullup resistor of 5 ohms from power to pin 13, it should work again.

- The old PCB had the MISO and MOSI signals placed wrong around the magnetometer. This has been changed on the Altium schematic; however, the new PCB hasn’t been ordered yet.

- The existing schematic uses MPU-9250 however the used part is the ICM-20948

- Processor – STM32F427VIT6 bought from Ali-express and hasn’t been tested yet.

Components Owned

- 4x STM32F424VIT6 - Microcontroller - IC1

- 5x MCP131T-300E/TT - Microchip supervisory circuit - IC2

- 4x FM25V - NVRAM - IC3 (Maybe Past shelf life)

- 4x ICM-20948 - IMU - IC4 (Maybe Past shelf life)

- 2x ICM-20608-G - IMU - IC5 (Apparently doesn't work), (Maybe Past shelf life)

- 4x MS56110 - Pressure sensor - IC6 (Maybe Past shelf life)

- 5x NCP163ASN - Voltage regulator - IC7

- 10x TJA1051 - CAN interface IC - IC8

- 5x 74LVC1G86SE - Logic Gates - IC9 (Maybe past shelf life)

- 5x 74LVC2G86DC - Logic gates - IC10 (Maybe past shelf life)

- 30x AP7365 - Voltage regulator - IC11

- 5x SM06B-SRSS - Right angle PCB connector - J1

- 5x 47346 - Micro USB connector - J2

- 40x SM06B-GHS-TB - Right Angle PCB Connector - J3, J7, J8, J9

- 5x PJS008 - Memory card connector - J4

- 5x TSW-106 - PCB header - J10, J11, J12

- 5x SSM-105 - Surface mount headers and wire housing - JP1

- 5x FA-20H - Epson crystals - Y1

- 4x 0ZCK0035FF2G - Resettable fuses - F1 (Maybe Past shelf life)

- 4x 0ZCJ0100FF2E - Resettable fuses - F2 (Maybe Past shelf life)

- 4x 0ZCJ0200FF2C - Resettable fuses - F3 (Maybe Past shelf life)

- 5x XP151A12A2MR - Power MOSFET - Q1

- 5x BSS209PW - MOSFET - Q2

- 5x APG1608 - Standard LEDs - LED1 (Maybe past shelf life)

- 5x LTST - Standard LEDs - LED2 (Maybe past shelf life)

- 10x LP5907 - Voltage regulator - U1, U2

- 2x LIS3MDLTR - Magnetometer - U3 - (Maybe Past shelf life)

- 5x CDBU70 - Schottky Diode - D1

- 5x PMEG2020CPASX - Schottky diode - D2

- 5x MMSZ5232BT1G - Zener Diode - D3

- 4x PMEG1030EH - Schottky Diode - D4

- 5x PMEG1030EJ - Schottky Diodes - D4

- 5x 1N4448X - General purpose diode - D5

- 47x CMP0603 - 0603 Thick film resistors 10k - R1, R2,R12,R13,R21,R42,R48,R49,R53

- 90x CRCW0603220R - 0603 Thick film resistors 220 ohm - R3, R20, R24, R25, R26, R27, R28, R29, R30, R31, R34, R35, R36, R37, R38, R39, R44, R45, R46

- 5x RCC0603 - 0603 Thick film resistor 1k - R4, R8, R8, R22, R23

- 10x SDR03EZ - 0603 Thick film resistor 220 ohm - R6, R7

- 18x ERJ-UP3D1003V - 0603 Thick film resistors 100k - R14, R15, R16, R17, R18

- 5x SDR03EZ - 0603 Thick film resistor - 120-ohm R19

- 15x ERA-3VEB1501V - 0603 Thin film resistor 1.5k - R32, R33, R40

- 4x CHP0603 - 0603 Thick film resistor 470 ohm - R43

- 19x SDR03EZPF1000 - 0603 Thick film resistors 100 ohm - R50, R51, R52

- 60x C1608X7S - 0603 Multilayer ceramic capacitor 2.2uF - C1, C7, C12, C13, C14, C17, C29, C22, C27, C36, C37, C38, C40, C47

- 90x C0603C104 - 0603 Multilayer ceramic capacitor 100nF - C2, C5, C6, C8, C9, C10, C11, C18, C19, C21, C23, C25, C28, C30, C31, C32, C35, C45, C46, C48

- 13x CGA3E3X7T0G - 0603 Multilayer ceramic capacitor 10uF - C15, C29, C43

- 10x GRM31CR6 - 1206 Multilayer ceramic capacitor 220uF - C16

- 2x EMF107B7 - 0603 Multilayer ceramic capacitor 470nF - C24

- 5x W2H13C1048 - Feed through capacitors - C26, C39

- 8x GCM188R71 - 0603 Multilayer ceramic capacitor 10nF - C33, C34

- 5x GRM31CC8 - 1206 Multilayer Ceramic capacitor 100uF - C41, C42

- 10x GRM31CR61A - 1206 Multilayer ceramic capacitor 100uF - C41, C42

- 4x JMK107AB - 0603 Multilayer Ceramic capacitor 10uF - C44

- 5x M50-3100545 - Headers and wire housing - wrong part ordered.

- 2x ESP-01 8266 Wi-Fi module to plug into SSM-105-L-DV

- 2x 5 - 3.3v level shifter

- 2x 5 - 1.8v level shifter

Components Missing

- J5, J14 - > 4 contact right angle surface mount PCB header -> SM04B - GHS - TB_LF__SN_ -

- J6, J13 - > 5 contact right angle surface mount PCB header -> SM05B-GHS-TB(LF)(SN))

- R47 - > 0603 surface mount 100 ohm - https://au.mouser.com/ProductDetail/ROHM-Semiconductor/ESR03EZPF1000?qs=493kPxzlxfJdLkJZpX4ByA%3D%3D

- R41 - > 0603 surface mount 680 ohm - https://au.mouser.com/ProductDetail/Panasonic/ERJ-1GJF6800C?qs=sGAEpiMZZMvdGkrng054t2cqbZCzJY7NzpXEN9eqq4TjsVyYOCLqng%3D%3D

- R10, R11 - > 0603 surface mount 1Mohm - https://au.mouser.com/ProductDetail/ROHM-Semiconductor/SFR03EZPJ105?qs=sGAEpiMZZMtG0KNrPCHnjW0BEZmmc%252BDp%252BI0%2Ff8pEXbW8XXIQvQyksA%3D%3D

- C3, C4 - > 0603 surface mount 8 pF - https://au.mouser.com/ProductDetail/TAIYO-YUDEN/MBASU063SCG080CFNA01?qs=dbcCsuKDzFXPU53IzeWGhA%3D%3D

- MS56110 - Pressure sensor - IC6 - https://au.mouser.com/ProductDetail/Measurement-Specialties/MS561101BA03-50?qs=%252BgKeJhng5iV%252BnCz6Qd5iDw%3D%3D

- 0ZCK0035FF2G - Resettable fuses - F1 - https://au.mouser.com/ProductDetail/Bel-Fuse/0ZCK0035FF2G?qs=SRYZG9HaIQ1hwWyCkUgYKw%3D%3D

- 0ZCJ0100FF2E - Resettable fuses - F2 - https://au.mouser.com/ProductDetail/Bel-Fuse/0ZCJ0100FF2E?qs=SRYZG9HaIQ16trCtSE8WyA%3D%3D

- 0ZCJ0200FF2C - Resettable fuses - F3 - https://au.mouser.com/ProductDetail/Bel-Fuse/0ZCK0035FF2G?qs=SRYZG9HaIQ1hwWyCkUgYKw%3D%3D

- LIS3MDLTR - Magnetometer - U3 - https://au.mouser.com/ProductDetail/STMicroelectronics/LIS3MDLTR?qs=hnsWAVXnr6EbLE48dGN9Pg%3D%3D