# QUT Autopilot – Handover Documentation

Last Updated: 11/10/2022

# Purpose of Project

- Create QUT’s own UAV Autopilot to use in subjects.

- Allows the team to order and get replacement on demands easier than purchasing pre-made versions

- Easier upgrades or modifications to existing model, rather than “hacking” pre-made ones.

# Reason for Project

- The platform that QUT EEI team is buying are constantly out of stock and may not have enough for the next year or so due to previous damages.

- New models will come out and old models will be discontinued, thus inconsistent with existing platforms that QUT already owns

  - This case will need to consider whether to buy whole new batch or just the case some would have to use older ones and some would get newer ones

- A lot of times the current platform is out of stock and may need to resort to another autopilot platform, thus another inconsistency.

# Current Build

- The current QUT Autopilot is based on the Pixhawk FMUv4 – PixRacer Design.

  - It is also the autopilot platform that QUT is mainly using for most of the subject

  - Additionally, is the latest one which provide an open-source schematic design

- Currently, first version of the PCB has been designed, and components have been purchased (Enough to create 5)

  - Designed using Altium

  - However, some sensors have already been used on three boards to test the connection.

  - Some components also have been used on the three boards

  - So, remaining would be enough to create 2 complete boards.

- Tested the sensors, where 3 out of the 4 sensors are receiving signals with sensible data.

  - ICM-20948 (9-Axis Sensors), Accel/Gyro/Mag. However, only getting Accel/Gyro Data.

  - LIS3MDLTR (Magnetometer). Working

  - MS5611xxx (Barometer). Working

  - ICM-20608-G (6-axis Sensor), Accel/Gyro. NOT WORKING

# Problems Encountered / Be Aware

- The schematic of FMUv4 – PixRacer have components that differ to the ones used in the QUT-Autopilot due to discontinued components or out of stock.

- The QUT-Autopilot IMU schematic uses ICM-20602, where the components acquired is ICM-20608-G. The pins have double checked to be the same, with very similar characteristics. So should not be a problem. If in doubt, please double check.

- Also do aware due to chip shortage, many chips are hard to get, especially the main processor chip, STM32F427VIT6. The one acquired was from ali-express, unsure of its authenticity / useability. This chip was not tested yet, due to the issue with sensors.

- Aware testings are made using Arduino, some libraries are grabbed from repositories as well as Arduino library manager.

  - Most libraries should be included in file path “T:\Techs\WIL_Wesley\QUT_UAV_Autopilot\FMUv4_Pixracer\Software\libraries”

    - Transfer this file to “Document\Arduino\libraries” and it should work

  - For library from Manager

    - Sparkfun 9DoF IMU Breakout (For IMC-20948)

- The Magnetometer sensor LIS3MDxx has the MISO and MOSI signals placed wrong, so the board cannot be applicable for full build (DO create a new board). Have not changed the signals around in Altium, just to be consistent with the error.

# Future Improvements / Suggestions

- Preferably re-design the board layout with larger spacings, larger board and several testing points to test the sensor’s signal as well as various other signals. This can help verify that the board connections are correct. (Through holes for jumper wires are recommended at least for the sensors, since it can be easily soldered and plug right into Arduino for testing).

- Maybe can design a temporary chip testing PCB to test sensors / chip integrity, and verify they are not broken.

- Maybe create a better Arduino software for testing SPI to sensors. Right now, using pre-made libraries to test, since limited understanding to SPI.